PRODUCT DATASHEET

OUTDOOR LIGHTING | Outdoor Wall MOUNTED

Outdoor lighting | Wall mounted light |IP65

Code# OTWM20013-8

Areas of Application

- Courtyard

- Park

- Villa

- Corridor

Product Features

- IP65

- Aluminum Die-casting lamp body, Opal PC Diffuser,

- Powder coating White/Black optional

- High luminous efficacy, high CRI

- Anti-glare, without flicker

- Easy and quick installation

Complete Product Data

Code#                                      	OTWM20013-8

Wattage                                    	8W

Installation Type                        	    Wall-Mounted

Dimension                                 	    105*120

Cutout                                   	 	/

IP Rating                                    	IP65

Luminous Flux                             		/

Beam Angle                               		Wide

Working Temperature                       		-25℃ ~55℃

Certificates                           		/

Warranty                                 		3 Years

Packing Size                               		/

Driver Data

Driver Brand                                	Tridonic, Osram, Philips, Lifud, Euchips, Eaglerise, etc.

Input Voltage                                	100-250VAC 50/60hz

Output Voltage                              	/

Output Current                              	/

Power Factor                                	>0.5 as standard; >0.9 optional

Efficiently                                   	87% @230v

IP Rating                                   	IP20

Dimmable                                		Non-Dimmable/Triac/0-10v/Dali Optional

Free Flicker                               		Yes

Certificates                               		CE, CB, CCC, ENEC

Warranty                                 		3/5 Years

Material&Housing Data

Light Body Color                            	   	White/Black

Reflector Color               		/

Light Body Material                         		Die-casting Aluminum

Photometric Data

LED chip brand                            		Cree/ Osram /Bridgelux/ Epistar etc.

Chip Type                                		SMD

Color Temperature                        		2700K, 3000K, 4000K, 6000K

CRI                                     		>80

Life time                                		>50000Hrs

Lumens efficacy	              		100-120lm/w

Dimensions&Installation

PLEASE FIND A QUALIFIED ELECTRICIAN FOR INSTALLATION. Please read the instructions before you install and use the outdoor wall-mounted light.

- Make sure the power is turned off in which you are installing the products.

- Fix the base with the screwdriver. (Fig 1)

- Connect the input terminal to the main power supply. (Fig2)

- Fix light body with tool. (Fig 3)

- Restore power at the source and the installation is complete.